- SHAFTLESS TYPE TROMMEL SCREEN

| NO. | ITEM & DESCRIPTION |  | DATA |
| --- | --- | --- | --- |
| 1.1 | Material to be Handled |  | compost (organic fraction of waste) |
|  | Bulk Density |  | 0.6 t/m3 |
|  | Moisture Content |  | < 20% |
|  | Maximum Particle Size |  | 6.5 mm |
|  | Capacity |  | 5 ton/hour |
| 1.2 | Equipment Name |  | shaftless type trommel screen |
|  | Material of Construction |  | carbon steel |
| 1.3 | Screening Drum Diameter |  | 1500 mm |
|  | Screening Drum Length |  | 3500 mm |
|  | Mesh Type |  | manganese steel punched plate |
|  | Mesh Size | Hole Opening 4 mm | round hole, hole center distance 15 mm, plate thickness 3mm |
|  | Inclination Angle |  | 0° (horizontal) |
|  | Rotation Speed |  | 20 rpm |
| 1.4 | Drive Arrangement |  | gear motor |
|  | Power Supply |  | 3 phase / 400V / 50 Hz |
|  | Type of Gear Motor |  | As per manufacturer |
|  | Power Rating |  | As per manufacturer |
| 1.5 | Rotating Brush |  | To be supplied and fitted on top of trimmer along the whole length of trommel |