ANNEX 1

TECHNICAL SPECIFICATIONS OF THE CONTAINER SKIP TRUCK

| Item | Specifications | Bidder's Information |
| --- | --- | --- |
| ENGINE | Number of cylinders: 6 in-line cylinders with turbocharger |  |
|  | Minimum power: 155 kW |  |
|  | Minimum torque: 750 Nm |  |
|  | Fuel system: Direct injection |  |
|  | Fuel: Diesel |  |
|  | Drive configuration: 4×2 or 6×4 |  |
|  | Minimum payload capacity: 5,500 kg (4×2) or 8,500 kg (6×4) |  |
|  | Cooling system: Water-cooled |  |
| TRANSMISSION | Manual gearbox with a minimum of 6 forward gears and rear differential lock |  |
|  | Single-disc clutch, with or without assistance (pneumatic, hydraulic or hydro-pneumatic) |  |
| SUSPENSION | Front: Reinforced semi-elliptic or parabolic suspension, consistent with the declared payload capacity |  |
|  | Rear: Bogie (tandem) suspension or equivalent reinforced semi-elliptic suspension |  |
| STEERING | Hydraulic power steering; right-hand drive (RHD) |  |
| BRAKES | Pneumatic braking system with drum or disc friction brakes on both front and rear axles |  |
|  | Anti-lock Braking System (ABS) mandatory |  |
| TYRES AND WHEELS | 11.00R22.5 tyres (or equivalent size with equal or higher load index), dual rear wheels |  |
| SAFETY | Fire extinguisher |  |
| HYDRAULIC SYSTEM | Hydraulic system for lifting and handling waste containers with a minimum capacity of 6 m³, operated through a Power Take-Off (PTO) |  |
| TRAINING | Training for 4 operators on operation and preventive maintenance of the equipment |  |
| AFTER-SALES SERVICE AND WARRANTY | Official manufacturer's representative with a branch in the province |  |
|  | Documentary evidence of the availability of spare parts (e.g. stock list, supply agreement or declaration from the local representative) |  |
|  | Minimum one-year manufacturer's warranty from the date of delivery/acceptance, supported by the manufacturer's official warranty certificate |  |

ANEXO 1

ESPECIFICAÇÕES TÉCNICA DE CAMIÃO PORTA-CONTENTOR

| Item | Especificações | Informações do concorrente |
| --- | --- | --- |
| MOTOR | Número de Cilindros: 6 cilindros em linha com turbo |  |
|  | Potência mínima: 155 kW |  |
|  | Torque mínimo: 750 Nm |  |
|  | Alimentação: Sistema de injecção |  |
|  | Combustível: diesel |  |
|  | Tracção: 4x2 ou 6x4 |  |
|  | Capacidade de Carga mínima: 5.500 kg (tracção 4x2) ou 8.500 kg (tracção 6x4). |  |
|  | Tipo de Refrigeração: água |  |
| TRANSMISSÃO | Caixa: manual, mínimo de 6 velocidades, bloqueio de diferencial traseiro |  |
|  | Embraiagem: monodisco, com ou sem assistência (pneumática, hidráulica ou hidro-pneumática) |  |
| SUSPENSÃO | Dianteira: semi-elíptica ou parabólica, reforçada de forma consistente com a capacidade de carga declarada |  |
|  | Traseira: tipo bogie (tandem) ou semi-elíptica reforçada equivalente |  |
| DIRECÇÃO | Assistida hidraulicamente; volante posicionado à direita |  |
| TRAVÕES | Sistema de travagem pneumático, com mecanismo de fricção a tambor ou a disco, à frente e atrás |  |
|  | Sistema antibloqueio de travões (ABS) obrigatório |  |
| PNEUS E RODAS | 11.00R22.5 (ou medida equivalente com índice de carga igual ou superior), rodado duplo no(s) eixo(s) traseiro(s) |  |
| SEGURANÇA | Extintor de incêndio |  |
| SISTEMA HIRAÚLICO | Para remoção e manuseamento de contentor, com capacidade mínima de 6 m³, comandado por tomada de força (PTO) |  |
| CAPACITAÇÃO | Capacitação de 4 pessoas sobre operação e manutenção preventiva do equipamento |  |
| ASSISTÊNCIA E GARANTIA | Representante oficial da marca com filial na província |  |
|  | Comprovativo documental de disponibilidade de peças sobresselentes (ex. lista de stock, acordo de fornecimento, ou declaração do representante local) |  |
|  | Garantia mínima de 1 ano a partir da entrega/aceitação, com certificado oficial do fabricante |  |