ANNEX 2

ECHNICAL SPECIFICATIONS OF THE TIPPER TRUCK – LOT 2

| Item | Specifications | Bidder's Information |
| --- | --- | --- |
| ENGINE | Minimum power: 86 kW at the manufacturer's rated engine speed |  |
|  | Minimum torque: 400 Nm at the manufacturer's rated engine speed |  |
|  | Engine displacement: 3,760–6,630 cm³ |  |
|  | Fuel system: Direct injection |  |
|  | Fuel: Diesel |  |
|  | Drive configuration: 4×2 or 4×4 (4WD) |  |
|  | Minimum payload capacity: 4,000 kg with a Gross Vehicle Weight (GVW) not exceeding 12,000 kg |  |
|  | Cooling system: Water-cooled |  |
|  | Type: Truck with tipper body |  |
| TRANSMISSION | Manual, synchromesh gearbox with a minimum of 6 forward gears |  |
|  | Clutch suitable for the transmission type |  |
| SUSPENSION | Front: Reinforced semi-elliptic leaf springs with shock absorbers |  |
|  | Rear: Reinforced semi-elliptic leaf springs with shock absorbers |  |
| STEERING | Hydraulic power steering |  |
| BRAKES | Pneumatic braking system |  |
|  | Disc and/or drum brakes on both front and rear axles |  |
|  | Parking/emergency brake with spring-actuated rear brake system |  |
| TYRES AND WHEELS | Single front tyres and dual rear tyres, with a load index appropriate for the vehicle's Gross Vehicle Weight (GVW) |  |
| SAFETY | Fire extinguisher |  |
| HYDRAULIC SYSTEM | Hydraulic tipping system for lifting the cargo body |  |
| BODYWORK | Reinforced tipper body suitable for the transportation of waste and other materials |  |
| TRAINING | Training for 4 operators on operation and preventive maintenance of the equipment |  |
| AFTER-SALES SERVICE AND WARRANTY | Official manufacturer's representative with a branch in the province |  |
|  | Documentary evidence of the availability of spare parts (e.g. stock list, supply agreement or declaration from the local representative) |  |
|  | Minimum one-year manufacturer's warranty from the date of delivery/acceptance, supported by the manufacturer's official warranty certificate |  |

ANEXO 2

ESPECIFICAÇÕES TÉCNICAS DA CAMIONETA BASCULANTE – LOTE 2

| Item | Especificações | Informações do concorrente |
| --- | --- | --- |
| MOTOR | Potência: mínimo de 86 kW no regime nominal indicado pelo fabricante |  |
|  | Torque: mínimo de 400 Nm no regime nominal do fabricante |  |
|  | Cilindrada: 3760 cc – 6630 cc |  |
|  | Alimentação: sistema de injecção |  |
|  | Combustível: diesel (gasóleo) |  |
|  | Tracção: 4x2 ou 4x4 (4WD) |  |
|  | Capacidade de carga: mínimo de 4.000 kg com Peso Bruto de Viatura (PBV) não superior a 12.000 kg |  |
|  | Tipo de refrigeração: água |  |
|  | Tipo: camião com carroçaria basculante |  |
| TRANSMISSÃO | Caixa: manual e sincronizada com mínimo de 6 velocidades |  |
|  | Embraiagem: sistema adequado ao tipo de transmissão |  |
| SUSPENSÃO | Dianteira: molas semi-elípticas (de lâmina), com amortecedores |  |
|  | Traseira: molas semi-elípticas reforçadas (de lâmina), com amortecedores |  |
| DIRECÇÃO | Assistida, hidráulica |  |
| TRAVÕES | Sistema pneumático |  |
|  | Travões de disco e/ou tambor à frente e atrás |  |
|  | Travão de estacionamento/emergência com mola traseira |  |
| PNEUS | Pneus: dianteiros de rodado simples e traseiros de rodado duplo, dimensionados com índice de carga adequado ao PBV do veículo |  |
| SEGURANÇA | Extintor de incêndio |  |
| SISTEMA HIDRÁULICO | Sistema basculante hidráulico para elevação da caixa de carga |  |
| CARROÇARIA | Caixa basculante reforçada, adequada para transporte de resíduos e materiais diversos |  |
| CAPACITAÇÃO | Formação de 4 operadores em operação e manutenção preventiva do equipamento |  |
| ASSISTÊNCIA E GARANTIA | Representante oficial da marca com filial na província |  |
|  | Evidência por parte do fornecedor de facilidade de aquisição de peças sobresselentes |  |
|  | Garantia contra defeitos de fabrico mínima de um ano |  |